Arduino Code

The Arduino reads the card, displays info, and plays sound:

cpp

CopyEdit

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <SoftwareSerial.h>

#include <DFRobotDFPlayerMini.h>

#include <SPI.h>

#include <MFRC522.h>

// LCD I2C

LiquidCrystal_I2C lcd(0x27, 16, 2); // Adjust address if needed

// DFPlayer Mini on pins 4 (RX), 5 (TX) Arduino side

SoftwareSerial dfSerial(4, 5);

DFRobotDFPlayerMini myDFPlayer;

// RFID pins

#define RST_PIN 9

#define SS_PIN 10

MFRC522 rfid(SS_PIN, RST_PIN);

// DFPlayer Reset pin (optional)

#define DFPLAYER_RESET_PIN 8

// Known RFID UIDs and names

String knownUIDs[] = {

"EA514D63"

};

String names[] = {

"Dinesh"

};

// Retry DFPlayer initialization

bool initDFPlayer() {

for (int i = 0; i < 5; i++) {

if (myDFPlayer.begin(dfSerial)) {

return true;

}

delay(1000); // Wait 1 sec before retry

}

return false;

}

void setup() {

delay(3000); // Let power stabilize

pinMode(DFPLAYER_RESET_PIN, OUTPUT);

digitalWrite(DFPLAYER_RESET_PIN, LOW);  // Hold DFPlayer in reset

delay(100);  // Reset pulse

digitalWrite(DFPLAYER_RESET_PIN, HIGH); // Release reset

delay(1000); // Wait for DFPlayer to boot

Serial.begin(9600);   // Serial to ESP32

dfSerial.begin(9600); // DFPlayer serial

SPI.begin();

rfid.PCD_Init();

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("Scan your ID...");

if (!initDFPlayer()) {

lcd.setCursor(0, 1);

lcd.print("DFPlayer Error");

Serial.println("Unable to initialize DFPlayer Mini!");

while (true); // Halt

}

myDFPlayer.volume(30); // Set volume (0-30)

}

void loop() {

if (!rfid.PICC_IsNewCardPresent() || !rfid.PICC_ReadCardSerial()) {

return;

}

// Read UID and convert to uppercase hex string

String uid = "";

for (byte i = 0; i < rfid.uid.size; i++) {

if (rfid.uid.uidByte[i] < 0x10) uid += "0";

uid += String(rfid.uid.uidByte[i], HEX);

}

uid.toUpperCase();

Serial.println("UID: " + uid);

bool matchFound = false;

for (int i = 0; i < 6; i++) {

if (uid == knownUIDs[i]) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Welcome,");

lcd.setCursor(0, 1);

lcd.print(names[i]);

Serial.println("Attendance marked for " + names[i]);

// Send matched UID and name to ESP32

Serial.print("MATCH:");

Serial.print(uid);

Serial.print(",");

Serial.println(names[i]);

myDFPlayer.play(1); // Play welcome sound

matchFound = true;

break;

}

}

if (!matchFound) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Unknown card");

Serial.println("Unknown UID");

// Send unknown info to ESP32

Serial.println("UNKNOWN");

myDFPlayer.play(2); // Play error sound (optional)

}

delay(3000); // Wait before next scan

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Scan your ID...");

}

ESP32 Code (Receives & Sends to Google Sheet)

cpp

CopyEdit

#include <WiFi.h>

#include <HTTPClient.h>

#define RXD2 16  // Arduino TX to ESP32 RX

#define TXD2 17  // Not used, but required for Serial2 init

const char* ssid = "*****";         // 🔁 Replace with your WiFi SSID

const char* password = "****";       // 🔁 Replace with your WiFi password

const char* serverName = "*****";     // 🔁 Replace with excelsheet link

void setup() {

Serial.begin(115200);

Serial2.begin(9600, SERIAL_8N1, RXD2, TXD2);  // Serial2 from Arduino

Serial.println("Connecting to WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi connected!");

}

void loop() {

if (Serial2.available()) {

String name = Serial2.readStringUntil('\n');

name.trim(); // Remove leading/trailing whitespace

Serial.println("Received from Arduino: " + name);

// Remove "MATCH:" prefix if present

if (name.startsWith("MATCH:")) {

name = name.substring(6);

}

// Remove everything before the first comma, including the comma

int commaIndex = name.indexOf(',');

if (commaIndex != -1) {

name = name.substring(commaIndex + 1);

}

Serial.println("Cleaned name: " + name);

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

String url = String(serverName) + name;

http.begin(url);

int httpResponseCode = http.GET();

if (httpResponseCode > 0) {

Serial.println("Data sent to Google Sheet!");

} else {

Serial.print("Error sending data: ");

Serial.println(httpResponseCode);

}

http.end();

} else {

Serial.println("WiFi disconnected!");

}

}

}